PRELIMINARY WORK

- What are the alternative and direct current, explain them briefly by giving example.

- Explain how the phase differences of two AC signals are measured on an oscilloscope by giving an example.

- Explain why phase difference is important.

- What is the way to reduce the phase difference (power factor) between current and voltage ? Explain it by giving an example.

- Explain how inductor and capacitor behaves in DC current.

- Solve the circuit in Figure 1 with the complex numbers, and writhe down them in their places (yellow area) in Table 1.

- Solve the circuit in Figure 1 with the phasor diagram, and writhe down them in their places (yellow area) in Table 1.

- Solve the circuit in Figure 3 with complex numbers only for R1=10kΩ, and write down them in their places (yellow area) in Table 2.

- Solve the circuit in Figure 3 with phasor diagram only for R1=10kΩ, and write down them in their places (yellow area) in Table 2.

- Adjust the circuit elements in Figure 5 as V1 (5V, 1kHz), R (kΩ), L (mH), C(nF), and write down vectoral and phasoral solutions in Table 3 (yellow area).

Explain how to calculate effective and mean value of an AC signal by giving an example.

NOTE: Come to the experiments by preparing the preliminary work as a report (with the report cover). Those who do not have a report will not be attended in the experiments.

ALTERNATIVE CURRENT (AC) CIRCUITS

Aim of Experiment: 1. Learning the current and voltage measurements in AC circuits.

2. Learning the measurement of phase difference.

3. Verifying the Superposition theorem.

4. Developing the student's ability to set up an experiment.

AC current circuit elements are resistor (R), inductance (L) and capacitor. AC ammeter and AC voltmeter are used to measure currents and voltages of these elements. AC ammeter and voltmeter measure the effective value (rms value) of the measured signal. While measuring a signal, it is important to choose the correct measuring range of the measuring tools for accuracy and safety.

EXPERIMENTAL PROCEDURE

- EXPERIMENT: Resistive Circuit

Set up the circuit given in Figure 1 below on the test board according to the element values in the adjacent table and set the frequency value of the V1(t)  voltage source to be 1kHz and the maximum value to be 5V.

| Elements | Values |
| --- | --- |
| V1(t) | 5.sin(wt) |
| R1 | 390Ω |
| R2 | 560Ω |

Figure 1. AC circuit consisting only of resistor elements

# After installing the circuit in Figure 1 on the board and turning on the voltage source, connect the CH1 input of the oscilloscope to the A-C terminal and the CH2 input of the oscilloscope to the B-C terminal, and draw the signs seen on the oscilloscope in Figure 2, and write downthe maximum values of the measured voltages in Table 1.

# Find the phase difference (ØV1-R2) between VAC and VBC by using oscilloscope and write   down it in Table 1

# Calculate the current flowing through the circuit using one of the VAC or   VBC voltages and draw it on Figure 2.

# Increase the R1 from 390Ω to 680Ω and write down phase difference between VBC and VAC voltages in Table 1, and comment on the phase difference..

Figure 2. Oscilloscope screen

Table 1. Results of experiment 1

| AC circuit consisting only of resistor elements |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| For R1=390Ω |  | For R1=680Ω |  | Complex |  | Phasor |  |
| VAC | V | VAC | V | VAC | V | VAC | V |
| VBC | V | VBC | V | VBC | V | VBC | V |
| ØV1-R2 |  | ØV1-R2 |  | ØV1-R2 |  | ØV1-R2 |  |

2.	EXPERIMENT: Circuit consisting of resistor and Capacitor (R-C)

| Eleman | Değeri |
| --- | --- |
| V1(t) | 5.sin(wt) |
| R1 | 10kΩ |
| C2 | 100nF |

| Elements | Values |
| --- | --- |
| V1(t) | 5.sin(wt) |
| R1 | 10kΩ |
| C2 | 33nF |

Figure 3. AC R-C circuit

# After installing the circuit in Figure 3 (for R1=10kΩ) on the board and turning on the voltage source, connect the CH1 input of the oscilloscope to the A-C terminal and the CH2 input of the oscilloscope to the B-C terminal, and draw the signs seen on the oscilloscope in Figure 4a, and write down the maximum values of the measured voltages in Table 2.

# Find the phase difference between VAC and VBC voltages for R1=10kΩ (ØV1-C2) with the help of osciloscope and write down it in Table 2.

# Repeat the step 2 to find phase difference for R1=5kΩ and R1=15kΩ, and follow how the phase difference changes on the oscilloscope. Then writhe down these phase differences in Table 2, and draw the signs seen on the oscilloscope in Figure 4a.

# Find the phase difference between VAB and VBC voltages for R1=10kΩ (ØR1-C2) with the help of osciloscope and write down it in Table 2.

# Repeat step 4 to find phase difference for R1=5kΩ and R1=15kΩ, and follow how the phase difference changes on the oscilloscope. Then writhe down these phase differences in Table 2, and draw the signs seen on the oscilloscope in Figure 4b.

Table 2. Results of experiment 2

| R-C circuit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1=5kΩ |  | R1=10kΩ |  | R1=15kΩ |  | Complex (R1=10kΩ) |  | Phasor (R1=10kΩ) |  |
| VAC | V | VAC | V | VAC | V | VAC | V | VAC | V |
| VBC | V | VBC | V | VBC | V | VBC | V | VBC | V |
| VAB | V | VAB | V | VAB | V | VAB | V | VAB | V |
| ØV1-C2 |  | ØV1-C2 |  | ØV1-C2 |  | ØV1-C2 |  | ØV1-C2 |  |
| ØR1-C2 |  | ØR1-C2 |  | ØR1-C2 |  | ØR1-C2 |  | ØR1-C2 |  |

Figure 4a. Oscilloscope screen

Figure 4b. Oscilloscope screen

- EXPERIMENT: Serial R-L-C circuit

| Elements | Values |
| --- | --- |
| V1(t) |  |
| R1 |  |
| C2 |  |
| L3 |  |

Figure 5. AC R-L-C circuit

# Design a series R-L-C circuit whose values you can assign yourself and set it up on the board.

# Connect oscilloscope probes to the voltage source and the resistor, and observe these two voltages from the oscilloscope. Then draw them in Figure 6, and write the maximum values of them in Table 3.

# Measure the phase difference between these two voltages and write down it in Table 3.

Figure 6. Oscilloscope screen

Table 3. Results of Experiment 3

| R-L-C circuit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Experimental Results |  |  |  |  |  | Complex (R1=10kΩ) |  | Phasor (R1=10kΩ) |  |
| VAC | V | VAC | V | VAC | V | VAC | V | VAC | V |
| VBC | V | VBC | V | VBC | V | VBC | V | VBC | V |
| VAB | V | VAB | V | VAB | V | VAB | V | VAB | V |
| ØV1-R1 |  | ØV1-R1 |  | ØV1-R1 |  | ØV1-R1 |  | ØV1-R1 |  |

# IMPORTANT NOTE

In order to be carried out the experiments properly, it is necessary to make preliminary works and to know the theoretical part of the methods well. Because in order to find the values such as current, voltage and resistance required in the experiment, you need to use the information in theory and rearrange the circuit in accordance with the theory.