PRELIMINARY WORK

- Why the voltmeter is parallel connected to a component? What kind of problems occurs when the voltmeter is connected in serial? Explain these questions shortly.

- How a resistor value can be read with help of color bands on the resistor?

- Investigate the working principle of analog and digital voltmeters.

- Investigate the working principle of ohm meters.

- What do Time/Div and Volt/Div knobs of oscilloscope do?

- How can we calculate the period, frequency, and maximum value of a signal on the oscilloscope?

- Can we measure the current of a component with help of an oscilloscope? If this one is possible how we can measure it?

- Calculate all current, voltage, and power value of elements in Figure 5 by using the kirchoff laws. Calculate the equivalent resistance (input resistance) value of the circuit from the voltage source (look at circuit from voltage source). Write down all calculations on your preliminary report and also fill in only the results of these calculations on their places in Table 1 (yellow area) before coming to the experiment.

- Calculate all current, voltage, and power value of elements in Figure 6 by using the kirchoff laws. Calculate the equivalent resistance (input resistance) value of the circuit from the voltage source (look at circuit from voltage source). Write down all calculations on your preliminary report and also fill in only the results of these calculations on their places in Table 2 (yellow area) before coming to the experiment.

- Calculate all current, voltage, and power value of elements in Figure 7 by using the kirchoff laws. Calculate the equivalent resistance (input resistance) value of the circuit from the voltage source (look at circuit from voltage source). Write down all calculations on your preliminary report and also fill in only the results of these calculations on their places in Table 3 (yellow area) before coming to the experiment.

NOTE: Come to the experiments by preparing the preliminary work as a report (with the report cover). Those who do not have a report will not be attended in the experiments.

DC, AC CURRENT, VOLTAGE and POWER MEASUREMENTS

Aim of Experiment: 1. Gaining the ability to establish circuits on the board

2. Learning to measure current, voltage, and power

3. Learning to use oscilloscope

4. Verifying Kirchhoff current and voltage laws

MEASUREMENT TOOLS

Voltmeter: The device that measures current, voltage and resistance is called AVO meter. There are two types of voltmeters, analog and digital voltmeters. In order to measure the voltage of any element in the circuit with AVO meters, it is necessary to bring the function selector switch (commutator) to the voltage level. The most dangerous situation for the voltmeter is to measure a voltage greater than the voltage specified on the stage when the commutator is on the low voltage stage. In such a case, the fuse of the voltmeter may burn out or the voltmeter may completely break down. As can be seen in Figure 2, the voltmeter is connected in parallel to the element whose voltage is to be measured. The internal resistance of the ideal voltmeter is infinite. In practice, the internal resistance of voltmeters is between 100MΩ and 1GΩ.

Ampermeter: In order to measure the current of any element in the circuit with AVO meters, it is necessary to bring the function selector switch (commutator) to the current level. The most dangerous situation for the ampermeter is to measure a current greater than the current specified on the stage when the commutator is on the low current stage. In such a case, the fuse of the ampermeter may burn out or the ampermeter may completely break down. As can be seen in Figure 4, the ampermeter is connected in series with the element whose current is to be measured.

Ohmmeter: In order to measure a resistance with AVO meters, it is necessary to bring the function selector switch (commutator) to the resistance measurement stage. The element whose resistance is to be measured must not be connected to the circuit or one of its legs must be disconnected from the circuit. The ohmmeter is connected in parallel to the element to be measured, such as a voltmeter.

Oscilloscope: The oscilloscope is actually a voltmeter and like it must be connected in parallel to the element to be measured. The difference of the oscilloscope from the voltmeter is that the oscilloscope also gives the shape information of the sign whose voltage is to be measured on its screen. By using this shape information, the maximum value, period and frequency of the signal can be measured. If a problem occurs in the signal at any time, it can also be observed and obtained with the help of oscilloscope.

Measurement Errors

Kirchoff ‘s Voltage Law (KVL): KVL depends upon the concept of a loop. A loop is any closed path through the circuit which encounters no node more than once. Essentially, to create a loop, start at any node in the circuit and trace a path through the circuit until you get back to your original node. The algebraic sum of all voltage differences around any closed loop is zero. Voltage polarities in the loop are based on assumed polarities of the voltage differences in the loop. As long as the assumed directions of the voltages are consistent from loop to loop, the final result of the analysis will reflect the actual voltage polarities in the circuit.

In acircuit, voltage measurement is made with the help of voltmeter and voltmeter is  connected to the measured element in parallel as in Figure 2.

Kirchoff's voltage law for the circuit below is as follows

-----------------------------------------------------------------------------

Figure 1. Kirchoff voltage law

Figure 2. Measurement of the voltage of any element in the circuit with a voltmeter

Kirchoff ‘s Current Law (KCL):  The algebraic sum of all currents entering and exiting a node must equal zero. That is, if we assign a mathematical sign (polarity) to each current, denoting whether they enter (+) or exit (-) a node, we can add them together to arrive at a total of zero, guaranteed.

Current measurement in the circuit is made with the help of ampermeter and the ampermeter is

connected to the element to be measured in series as in Figure 4.

Kirchoff's law of currents in the circuit left is as

follows for nodes A and B

--------------------------------------------------------

Figure 3. Kirchoff ‘s current law

Figure 4. Measurement of the current of any element in the circuit with an ampermeter

EXPERIMENTAL PROCEDURE

- EXPERIMENT : Measurement of Resistance, Current and Voltage for Voltage Divider Circuit

| Elements | Values |
| --- | --- |
| E1 | 5V |
| R2 | 1kΩ |
| R3 | 100Ω |
| R4 | 180Ω |

Figure 5. Resistance and voltage measurement circuit (voltage divider circuit)

- Set up the circuit given in Figure 5 above according to the values in the table. To set up the circuit, find the values of the resistors with the help of an ohmmeter or by reading color codes of the resistors.

- After setting up the circuit, find the voltage falling on all resistors with the help of a voltmeter (in DC step) and write it in the appropriate place (colors will guide you) in Table 1.

- Measure the resistance value seen from the voltage source with the help of an ohmmeter and write it in the appropriate place in Table 1.

- Calculate the Absolute and Relative Error values for Voltage and Resistance measurements and write them in the appropriate place in Table 1.

- If the circuit in Figure 5 satisfies Kirchoff's voltage law, prove it according to the voltage values

- you measured

Table 1. Results obtained according to the circuit in Figure 5

| Results of Measurements of Voltage and Power |  |  |  | Results of Preliminary Questions |  |  |  | Absolute Error(V) | Relative Error(V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VR2 | V | PR2 | w | VR2 | V | PR2 | w |  |  |
| VR3 | V | PR3 | w | VR3 | V | PR3 | w |  |  |
| VR4 | V | PR4 | w | VR4 | V | PR4 | w |  |  |
| Result of Current Measurement |  |  |  |  |  |  |  | Absolute Error(V) | Relative Error(V) |
| I1 | A |  |  | I1 | A |  |  |  |  |
| Input Resistance |  |  |  | Results of Preliminary Questions |  |  |  | Absolute Error(V) | Relative Error(V) |
| Rgiriş | Ω | P | w | Rgiriş | Ω | P | w |  |  |

| Elements | Values |
| --- | --- |
| E1 | 5V |
| R2 | 1kΩ |
| R3 | 100Ω |
| R4 | 180Ω |

Figure 6. Current and voltage measurement (current divider circuit)

- Set up the circuit given in Figure 6 above according to the values in the table. Measure the currents and voltages in all branches using a voltmeter and ammeter (in DC range) and write them in the appropriate place in Table 2.

- Measure the resistance value seen from the voltage source with the help of an ohmmeter and write it in the appropriate place in Table 2.

- Calculate the power consumed by each element and write it in the appropriate place in Table 2.

- Calculate the Absolute and Relative Error values for Voltage and Resistance measurements and write them in the appropriate place in Table 2.

- If the circuit in Figure 6 satisfies Kirchoff's law of currents, prove it according to the current values you measured..

Table 2. Results obtained according to the circuit in Figure 6

| Results of Voltage Measurements |  |  |  |  |  | Results of Preliminary Questions |  |  |  |  | Absolute Error (I) | Relative Error(I) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VR2 | V |  |  |  |  | VR2 | V |  |  |  |  |  |
| VR3 | V |  |  |  |  | VR3 | V |  |  |  |  |  |
| VR4 | V |  |  |  |  | VR4 | V |  |  |  |  |  |
| Results of Current Measurements |  |  |  |  |  | Results of Preliminary Questions |  |  |  |  | Absolute Error (I) | Relative Error(I) |
| I1 | A |  | Güç |  |  | I1 | A |  | Güç |  |  |  |
| I2 | A |  | PR2 |  | w | I2 | A |  | PR2 | w |  |  |
| I3 | A |  | PR3 |  | w | I3 | A |  | PR3 | w |  |  |
| I4 | A |  | PR4 |  | w | I4 | A |  | PR4 | w |  |  |
| Input Resistance |  |  |  |  |  | Results of Preliminary Questions |  |  |  |  | Absolute Error (I) | Relative Error(I) |
| Rinput |  | Ω |  | P | w | Rinput |  | Ω | P | w |  |  |

- EXPERIMENT : Using Oscilloscope

| Elements | Values |
| --- | --- |
| V1 | 5V (1kHz) |
| R2 | 1kΩ |
| R3 | 100Ω |
| R4 | 180Ω |
| R5 | 180Ω |

Figure 7. Alternative circuit for Oscilloscope

- Set up the circuit in Figure 7 on the board according to the element values in the table. The voltage source is alternative, please make sure that the maximum value of 1KHz frequency is 5V.

- Connect one probe of the oscilloscope to the A-C and the other probe to the B-D terminals, adjust the Time/Div and Volt/Div buttons of the oscilloscope to the appropriate position in order to see the signals on the screen. Draw the signal shapes you see on the oscilloscope screen in Figure 8.

- Calculate the maximum values, peak-to-peak values, effective value, period and frequency of the signals you see on the oscilloscope and write them in the appropriate place in Table 3.

- Measure the resistance value seen from the voltage source with the help of an ohmmeter and write it in the appropriate place in Table 3.

- Arrange the knob of AVO meterr for AC signal measurement, and measure the current and voltage of all the elements in the circuit and write the values you have measured in the appropriate places in Table 3.

- If the circuit in Figure 7 satisfies Kirchoff's law of currents and voltages, prove it according to the current and voltage values you measured.

Table 3. Results obtained according to the circuit in Figure 7

| Results of AC Voltmeter |  |  |  | Results of Preliminary Questions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VR2 | V | I1 | A | VR2 | V | I1 | A |
| VR3 | V | I2 | A | VR3 | V | I2 | A |
| VR4 | V | I3 | A | VR4 | V | I3 | A |
| VR5 | V |  |  | VR5 | V |  |  |
| Input Resistance, Period, Frequency,Peak to Peak Value, and Max value |  |  |  | Results of Preliminary Questions |  |  |  |
| Rinput | Ω |  |  | Rinput | Ω |  |  |
| VP-P |  |  |  |  |  |  |  |
| Vmax |  |  |  |  |  |  |  |
| Vet |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |
| f |  |  |  |  |  |  |  |

Figure 8. Oscilloscope screen

# IMPORTANT NOTE

# In order to be carried out the experiments properly, it is necessary to make preliminary works and to know the theoretical part of the methods well. Because in order to find the values such as current, voltage and resistance required in the experiment, you need to use the information in theory and rearrange the circuit in accordance with the theory.