PRELIMINARY WORK

- Explain how an Ampermeter, voltmeter and ohmmeter are connected to an electrical circuit element.

- In the circuit in Figure 1, show how an ampermeter is connected to measure the current flowing through the R2 resistor by drawing, show how a voltmeter is connected to measure the voltage value across the R3 resistor by drawing, and show how an ohmmeter is connected to measure the resisance of the circuit when looked at from the green resistor R by drawing.

- Explain when the Thevenin, Norton and superposition methods are used.

- What is the common step in Thevenin and Norton methods?

- Which step does separate the Thevenin and Norton method from each other?

- If the Thevenin equivalent circuit of a circuit is known, how can the Norton equivalent

circuit be obtained using this Thevenin equivalent circuit?

- Calculate the parameters required to set up the Thevenin and Norton equivalent circuits

of the circuit in Figure 1.

- Using the superposition method, calculate the current flowing through R in Figure 1.

- Explain what the maximum power transfer is and under which conditions maximum

power can be transferred to the load resistor.

- Using the Thevenin and Norton method for Figure 8, set up the Thevenin and Norton

equivalent circuit of the circuit, and calculate the required element values in terms of Ry.

Then write down them in their places (yellow area) in Table 1.

- Using superposition method for Figure 8, calculate the currents of Ry. Then write down

them in their places (yellow area) in Table 3.

NOTE: Come to the experiments by preparing the preliminary work as a report (with the

report cover). Those who do not have a report will not be attended in the

experiments.

THEVENİN, NORTON TEOREMS and MAXIMUM POWER TRANSFER

Aim of Experiment: 1. Verifying Thevenin ve Norton Teorems

2. Verifying Maximum Power Transfer

3. Verifying Superpozision Teorem

4. Developing the student's ability to set up an experiment

Thevenin and Norton methods can be applied to any circuit containing linear circuit elements. The stage of creation of the Thevenin equivalent circuit of any circuit according to a certain element or set of elements is shown below in Figure 1. Thevenin equivalent circuit consists of a voltage source and the Thevenin equivalent resistance (RTh), which represents its internal resistance, and a resistor (R) in series with these elements. Norton equivalent circuit consists of a current source, a parallel equivalent resistor (RN) representing the internal resistance of the current source, and a resistor (R) parallel to these elements.

The Thevenin and Norton equivalent circuits of a circuit given in Figure 1 are seen just below of Figure 1. The Thevenin equivalent circuit of the rest of the circuit (shown by the dashed red lines) according to the green resistor R appears on the left, and the Norton equivalent circuit on the right.

As can be seen in Figure 1, while forming the Thevenin equivalent circuit of the main circuit according to the green resistor R, all the circuit elements in the red dashed line are represented by a voltage source (ETh) and a resistor (RTh) in series with it (can be seen in the blue dashed line). Similarly, while constructing the Norton equivalent circuit, all of the circuit elements in the dashed line are represented by a current source (IN) and a parallel resistor (RN) (can be seen in the brown dashed line).

a) Thevenin equivalent circuit

b) Thevenin equivalent circuit    	c) Norton equivalent circuit

# Figure 1. Thevenin and Norton eequivalent circuits of any electrical circuit

After the equivalent circuits are constructed, the current passing through the resistor R and the voltage dropping on it are found as follows:

For Thevenin method:	          For Norton method:

As can be seen from the expressions in the circuit, the current and voltage of the R in the main circuit and the current and voltage values of the R in the equivalent circuits are the same. If they are different, the calculations were made wrong.

Example :

Figure 2. Sample circuit

The equivalent resistance (RTh) of the circuit according to R in Figure 2 can be found to

form the Thevenin equivalent circuit as follows.
.

Figure 3. Calculation of Thevenin and Norton equivalent resistance

To find the value of the voltage source in the Thevenin equivalent circuit, the circuit is drawed as Figure 4, and the voltage seen from the ends of the removed R element is found by calculation. In order to measure this voltage (VTh), a voltmeter is connected to these terminals whose R resistance is removed. The connection can be seen in the figure on the right.

Figure 4. Finding the Thevenin voltage via calculation and measurement

According to these calculated values, the Thevenin equivalent circuit is formed as follows, and the current passing through the R resistor and the voltage dropping on it are calculated as on the right side.

Figure 5. Thevenin equivalent circuit of the sample circuit

Similarly, to create the Norton equivalent circuit, the following equivalent circuit is formed and the short-circuit current IN is calculated with the help of this circuit. In order measure current IN an ampermeter is directly connected to these terminals, where the R resistance is removed. The connection can be seen in the figure on the right.

Figure 6. Finding the Norton current via calculation and measurement

If the current I2 is negative, it only means that the direction shown in the above circuit is reversed, in fact, the current I2 flows in the opposite direction of the one shown. According to this short circuit current value IN, Norton equivalent circuit is formed as follows.

Figure 7. Norton equivalent circuit of the sample circuit

As can be seen from the results, the circuits formed as a result of Thevenin and Norton equivalent circuits have the same currents for the R and as a result, Same voltage values drop on the R in the equivalent circuits.

EXPERIMENTAL PROCEDURE

1. EXPERIMENT : Thevenin Method

Using the element values on the table (on the right side) set up the circuit given in Figure 8 on the breadboard.

| Elements | Values |
| --- | --- |
| E1 | 10V |
| E2 | 5V |
| R1 | 100Ω |
| R2 | 270Ω |
| Ry | 180Ω |

# Figure 8. Electrical circuit of experiment 1

| In the first step, the values found by measurement during the experiment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| VRy |  | V |  |  |  |  |
| IRy |  | A |  |  |  |  |
| Thevenin Method |  |  | Results of step 4 |  | Results of Preliminary report |  |
| VTh | V |  | VRy | V | VTh | V |
| RTh | Ω |  | IRy | A | RTh | Ω |
| Norton Method |  |  | Results of step 8 |  | Results of Preliminary report |  |
| IN | A |  | VRy | V | IN | V |
| RN | Ω |  | IRy | A | RN | Ω |

Table 1. Measured and Calculated results with respect to Experiment 1

# After installing the circuit on breadboard, and turning on the voltage source, connect a voltmeter to A-B terminals, an ampermeter to B-C terminals. Then write the current and voltage values of Ry on the amperemeter and the voltmeter in the appropriate place in Table 1 (colors will guide you).

# To form the Thvenin equivalent circuit of Figure 8 according to Ry as in Figure 9; RTh is needed to find via measurement. To do this extract the sources in the circuit and find the RTh via measuring it from A-B terminals

# To find VTh, reinstall the circuit on the breadboard for VTh and measure VTh voltage value by using a voltmeter.

- Using the measured values, reinstall the Thevenin equivalent circuit of Figure 8 on the breadboard as seen in Figure 9. Then measure current and voltage values of Ry and write on the section of Results of  step 4 in Table 1. Also find these current and voltage values by solving this circuit.

- Comment the results by comparing the values you have calculated in the preliminary report with the results you have measured.

Figure 9. Thevenin equivalent circuit

2. EXPERIMENT : Norton Method

# To form the Norton equivalent circuit of Figure 8 according to Ry as in Figure 10; RN is needed to find via measurement. To do this extract the sources in the circuit and find the RN via measuring it from A-B terminals.

# To find the IN, reinstall the circuit on the breadboard for IN and measure IN current value by using a amperemeter.

# Using the measured values, reinstall the Norton equivalent circuit of Figure 8 on the breadboard as seen in Figure 10. Then measure current and voltage values of Ry and write on the section of Results of step 8 in Table 1. Also find these current and voltage values by solving this circuit.

- Comment the results by comparing the values you have calculated in the preliminary report with the results you have measured.

Figure 10. Norton equivalent circuit

- EXPERIMENT: Maximum Power Transfer

# Install the circuit in Figure 8 on the breadboard and adjust the value of the Ry resistor as 33Ω, 75 Ω, 180Ω ve 270Ω, find every dissipated power value of Ry via measurement and write it in Table 2.

# Comment what value Ry in Figure 8 sould take to draw maximum power from the source.

Table 2. Dissipated Powers on Ry

| Ry | VRy | PRy |
| --- | --- | --- |
| Ry=33Ω |  |  |
| Ry=75Ω |  |  |
| Ry=180Ω |  |  |
| Ry=270Ω |  |  |

- EXPERIMENT: Superposition Theorem:

# Design a circuit with three resistors and two voltage sources and determine a resistor Ry. Make appropriate adjustments on the circuit to find the current that only the first voltage source will pass through the Ry resistor. After making the appropriate adjustments on the circuit, measure the flowing current from the B-C terminals and write it in Table 3.

# Make appropriate adjustments on the circuit to find the current that the just second voltage source will pass through the resistor Ry. After making the appropriate adjustments on the circuit, measure the flowing current from the B-C terminals and write it in Table 3.

Table 3. Results of superposition method

|  | E1 | E2 | Total Current |
| --- | --- | --- | --- |
| IRy |  |  |  |
| Results of Preliminary work |  |  |  |

# IMPORTANT NOTE

# In order to be carried out the experiments properly, it is necessary to make preliminary works and to know the theoretical part of the methods well. Because in order to find the values such as current, voltage and resistance required in the experiment, you need to use the information in theory and rearrange the circuit in accordance with the theory.