PRELIMINARY WORK

- Set up the circuit in Figure 1 according to the values in the adjacent table. Calculate the all of the node voltages and write down them in their places (yellow area) in Table 1.

- Calculate all currents in the circuit and write down them in their places (yellow area) in Table 2.

- Set up the circuit in Figure 3 according to the values in the adjacent table. Then find currents passed by each source through the resistor R2 by applying the superposition method, and write down them in their places (yellow area) in Table 2.

NOTE: Come to the experiments by preparing the preliminary work as a report (with the report cover). Those who do not have a report will not be attended in the experiments.

NODE VOLTAGES and SUPERPOSITION METHODS

Aim of Experiment: 1. Verifying Node Voltages Method

2. Verifying Superpozision Teorem

3. Developing the student's ability to set up an experiment

# Node Voltages Method

It is the method that facilitates the circuit solution if there are many current sources and many branches parallel to each other in the circuit. The node voltages method is based on Kirchhoff's current law.

# Superposition Method

The superposition method examines the current passing through the branches of a circuit separately according to each source. In this method, circuit solutions are made only when a independent source is active. The other independent sources are disabled so that the effect of each source on the currents in the circuit branches can be examined separately. The solution of the total circuit can be obtained as the sum or difference of the currents obtained in the superposition method (taking into account the directions of the currents).

EXPERIMENTAL PROCEDURE

- EXPERIMENT : Node Voltages Method

| Elements | Values |
| --- | --- |
| E1 | 10V |
| R1 | 1kΩ |
| R2 | 2,2kΩ |
| R3 | 4,7kΩ |
| R4 | 3kΩ |

Figure 1. Experiment of node voltages method

- Set up the circuit in Figure 1 and bring the value of voltage source to 10V.

Figure 2. Nodes and node voltages of the circuit

- In the circuit in Figure 2, connect the voltmeter between Vd1 and the Reference point and write down the measured voltage value in Table 1.

- In the circuit in Figure 2, connect the voltmeter between Vd2 and the Reference point and write down the measured voltage value in Table 1.

- In the circuit in Figure 2, connect the voltmeter between Vd3 and the Reference point and write down the measured voltage value in Table 1.

- In the circuit in Figure 2, connect the voltmeter between Vd1 and Vd2 and write down the measured voltage value in Table 1.

- In the circuit in Figure 2, connect the voltmeter between Vd1 and Vd3 and write down the measured voltage value in Table 1.

Table 1. Obtained voltage results according to the node voltages method

| Experimental Results of Node Voltages Method |  |  |
| --- | --- | --- |
| Measurement of Node Voltages | Volt | Volt |
| Vd1 - Reference Point |  |  |
| Vd2 - Reference Point |  |  |
| Vd2 - Reference Point |  |  |
| Vd1 - Vd2 |  |  |
| Vd2 - Vd3 |  |  |

# 7. Using the amperemeter, measure the current of R1 (I1), and write down in Table 2. Using the measured Vd1 -Referance Point in Table 1, calculate the same I1 current and write it down in Table 2.

# 8.  Using the amperemeter, measure the current of R2 (I2), and write down in Table 2. Using the measured Vd2 - Vd3 in Table 1, calculate the same I2 current and write it down in Table 2.

# 9. Using the amperemeter, measure the current of R3 (I3), and write down in Table 2. Using the measured Vd2 - Referance Point in Table 1, calculate the same I3 current and write it down in Table 2.

10. Compare your results of preliminary work (in the yellow area in Tables 1 and 2) with the results you obtained as a result of the experiment. If the results are different, discuss the reason for this difference.

| Experimental Results of Node Voltages Method |  |  |  |
| --- | --- | --- | --- |
| Currents | Measurements (Amper) | Calculation (Amper) | Amper |
| I1 |  |  |  |
| I2 |  |  |  |
| I3 |  |  |  |

Table 2. Obtained current results according to node voltages method

# EXPERIMENT: : Superposition Method

| Elements | Values |
| --- | --- |
| E1 | 10V |
| E2 | 5V |
| R1 | 100Ω |
| R2 | 560Ω |
| R3 | 1kΩ |
| R4 | 1kΩ |
| R5 | 100Ω |

Figure 3. A Sample circuit for superposition method

- Her When both sources are in the circuit, measure current values of R1 and R2 with an amperemeter, and write them down in Table 2.

- By extracting the E2 voltage source, measure current values that only the E1 voltage source passes through R1 and R2 with an amperemeter, and write them down in Table 2.

- By extracting the E1 voltage source, measure current values that only the E2 voltage source passes through R1 and R2 with an amperemeter, and write them down in Table 2.

- Compare your results of preliminary work (in the yellow area in Tables 3) with the results you obtained as a result of the experiment. If the results are different, discuss the reason for this difference.

- Deney öncesi hazırlık sonuçlarınızı (Tablo 3’de sarı bölgedeki), deney sonucunda elde ettiğiniz sonuçları karşılaştırınız. Sonuçlar farklı ise bu farkın nedenini tartışınız.

Table 3. Obtained results according to Figure 3

| Superposition Method Results |  |  |  |
| --- | --- | --- | --- |
| Only E1 is on | Only E2 is on | Only E1 is on | Only E2 is on |
| IE1-R1= | IE2-R1= | IE1-R1= | IE2-R1= |
| IE1-R2= | IE2-R2= | IE1-R2= | IE2-R2= |
| E1 and E2 are on |  | E1 and E2 are on |  |
| IR1= |  | IR1= |  |
| IR2= |  | IR2= |  |

# IMPORTANT NOTE

In order to be carried out the experiments properly, it is necessary to make preliminary works and to know the theoretical part of the methods well. Because in order to find the values such as current, voltage and resistance required in the experiment, you need to use the information in theory and rearrange the circuit in accordance with the theory.