PRELIMINARY WORK

- Expain Maximum Power transfer theorem for AC and DC circuits.

- In general, given a certain source impedance, what load impedance will achieve maximum load power (in AC circuits)?

- Will achieving maximum load power also achieve maximum efficiency? Explain.

- Solve all the steps of the experiment theoretically and find the required impedance value for maximum power transmission and record it in the yellow fields in the tables?

NOTE: Come to the experiments by preparing the preliminary work as a report (with the report cover). Those who do not have a report will not be attended in the experiments.

AC MAXIMUM POWER TRANSFER

Aim of Experiment: 1. Verifying the maximum power theorem in AC Circuits.

2. Developing the student's ability to set up an experiment.

- Maximum Power Transfer

In this experiment, maximum power transfer to the load will be examined for the AC circuits. Both the load’s resistive and reactive components will be independently varied to discover their effect of load power and determine the values required for maximum power transfer.

In the DC circuits, maximum power transfer is achieved by setting the load resistance equal to the source’s internal resistance. This is not true for the AC case. Instead, the load should be set to complex conjugate of the source impedance, the complex conjugate having the same magnitude as the original but with the opposite sign for the angle. By using the complex conjugate, the load and source reactive components will cancel out leaving a purely resistive circuit similar to the DC case. When calculating the true load power, care must be taken to remember that the load voltage appears across a complex load impedance. Only the real portion of this voltage appears across the resistive component, and only the resistive component dissipates power

EXPERIMENTAL PROCEDURE

Figure 1. Experiment of maximum power transfer

# Build the circuit of figure 1 using R=1kΩ and L=10mH, but leaving off the load components. Replace the voltage source with a 50Ω resistor and determine the effective source impedance at 10 kHz by calculating it. Record this value in Table 1, including both magnitude and phase. Determine the load impedance which should achieve maximum power transfer according to the theorem and record in Table 1. Finally, determine values for Rload and Cload to achieve this load impedance and record in Table 1, also copying the resistance value to the first Rload entry of Table 2.

Table 1. Data Table

Table 2. Data Table

# Replace the 50Ω resistor with the voltage source. Insert the decade resistance box in the position of RLoad and set it to the value calculated in Table 1. For CLoad, use the value calculated in Table 1. Use multiple capacitors if necessary to achieve a close value. Set the generator to 10 volts peak at 10 kHz, making sure that the amplitude is measured on the oscilloscope with the generator loaded by the circuit.

# Measure the magnitude of the load voltage and record in Table 2. Also compute the expected load voltage from theory and the load power based on the measured load voltage and record in Table 2 Repeat these measurements and calculations for the remaining load resistance values in the table.

Table 3. Data Table

# Return the decade box to the value calculated in Table 1. For Cload, insert the first capacitor listed in Table 3. Repeat step four for each load capacitance in Table 3, calculating and recording the required results using Table 3.

# Generate a plot of Pload with respect to Rload and another of Pload with respect to Cload.

# IMPORTANT NOTE

In order to be carried out the experiments properly, it is necessary to make preliminary works and to know the theoretical part of the methods well. Because in order to find the values such as current, voltage and resistance required in the experiment, you need to use the information in theory and rearrange the circuit in accordance with the theory.